INSTRUCTIONS:

- Edit name and reg # in header before printing this assignment.

- Then solve this assignment and scan using CamScanner.

- Also add your neat working on white printer pages.

- Submit L1F23BSCS0000.PDF File on portal.

Question#1

The values of various registers before executing each of the following instructions are as follows: DS:0xA1B9	SI:0x0024	DI:0x0032	BP: 0x0110	SS:0x3C80

AX:0xABCD BX:0x12C2	CX:0xFEED	DX:0x2032	ES:0x1AB0

  - Execute the following instructions and update the registers and memory accordingly. Also, calculate physical addresses in the hexadecimal number system. (10 Marks)

| Instructions | Physical Address | Registers |
| --- | --- | --- |
| MOV [BX+SI], CX | A2E76 |  |
| MOV [BP+DI+0x0016], DX | 3C958 |  |
| MOV AX, [BX+DI] | A2E84 | AX= 00 00 |
| MOV [BX+SI+9D], CX | A2E7F |  |
| MOV DX, DS:[BP+SI+11BCH] | A2E80 | DX= 00 FE |

Memory

|  |  |  | 0x0 | 0x1 | 0x2 | 0x3 | 0x4 | 0x5 | 0x6 | 0x7 | 0x8 | 0x9 | 0xA | 0xB | 0xC | 0xD | 0xE | 0xF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x1AB0 | : | 0x0120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x1AB0 | : | 0x0130 |  |  |  | 50 | 56 | 78 | AB |  |  |  |  |  |  |  |  |  |
| 0x1AB0 | : | 0x0140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x3C80 | : | 0x0150 |  |  |  |  |  |  |  | 20 | 89 |  |  |  |  |  | AB | CD |
| 0x3C80 | : | 0x0160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0xA1B9 | : | 0x12E0 |  |  |  |  |  |  |  | 10 | 67 |  |  |  |  |  | 40 | 92 |
| 0xA1B9 | : | 0x12F0 | 66 | F0 | 50 | 30 | 12 | 23 | 45 |  |  |  |  |  |  |  |  |  |
| 0xA1B9 | : | 0x1300 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EE | 89 |

  - Write the status of various flags after the execution of the ADD instruction. (6 Marks)

| MOV AL, 0x80 | CF | ZF | SF | OF | PF | AF |
| --- | --- | --- | --- | --- | --- | --- |
| ADD AL, AL | 1 | 1 | 0 | 1 | 1 | 0 |

Question#2 (3 + 3 + 3 Marks)

Write set of instructions to store 0x0BAD in memory at physical address 0x07DE1 using the following addressing modes one by one.

- Direct Addressing Mode

- Register Indirect Addressing Mode

- Register relative base plus index addressing mode.

| Part-A | Part-B | Part-C |
| --- | --- | --- |
| .MODEL SMALL .STACK 100H .DATA .CODE MOV AX, 07DEH MOV DS,AX MOV [0001H],0X0BAD | .MODEL SMALL .STACK 100H .DATA .CODE MOV AX, 07DEH MOV DS,AX MOV BX, 0001H MOV [BX],0X0BAD | .MODEL SMALL .STACK 100H .DATA .CODE MOV AX, 07DEH MOV DS,AX MOV BX, 0001H MOV SI,0000H MOV [BX + SI],0X0BAD |

Review the following instructions and fill in the table accordingly.

- Write TRUE for VALID instructions only?

- In the case of INVALID, write FALSE and also give reasons.

NOTE: Reasons based on rules or addressing modes. Avoid lengthy discussion.

| Sr# | Instructions | Valid/ Invalid | Reasons for INVALID instructions |
| --- | --- | --- | --- |
| 1. | MOV 0X1234, AX | INVALID | BECAUSE WE CAN’T MOVE THE VALUE IN AX INTO A CONSTANT LITERAL SO IN ORDER TO MOVE DATA FROM AX REGISTER TO A SPECIFIC MEMORY ADDRESS WE SHOULD ROUND IT WITH BRACKETS LIKE: MOV [OX1234], AX |
| 2. | MOV AX, [CX] | INVALID | BECAUSE ASSEMBLER ONLY ALLOWS BASE AND INDEX REGISTER AS MEMORY ADDRESS INSIDE BRACKETS BUT CX IS A GENERAL PUPOSE REGISTER USED FOR ARITHMETIC/DATA OPERATIONS NOT FOR ADDRESSING |
| 3. | MOV AL, [BL + 4] | INVALID | BECAUSE THE BL IS OF 8-BIT AND ONLY 16-BIT (BASE /INDEX) REGISTERS CAN HOLD MEMORY ADDRESS. |
| 4. | ADD DX, [BX+DX+2] | INVALID | BECAUSE IN 8086 ARCHITECTURE, AS I SAID ONLY CERTAIN REGISTERS CAN BE USED INSIDE [] AS ADDRESS REGISTER AND DX IS NOT ONE OF THEM. |
| 5. | MOV CS, AX | INVALID | ACCORDING TO RULES THE CS REGISTER CAN’T BE A DESTINATION OPERAND. Because CS determines which segment the CPU fetches instructions from. If you were allowed to modify it directly using MOV, the processor could jump to a completely different code segment in the middle of execution, WHICH BREAKS THE INSTRUCTION FLOW. |
| 6. | ADD NUM1, NUM2 | INVALID | both operands cannot be memory locations. At least one operand must be a register or an immediate value. The 8086 processor can’t perform memory to memory arithmetic directly,it can work between registers and memory/immediate |
| 7. | MOV DS, 0X720 | INVALID | Because we can’t assign immediate value in any segment register |
| 8. | ADD AX, BL | Invalid | Because the according to the rules the size of both operands should be the same but here the AL is of size 8-bit while AX is of 16-bit register so its not allwowed |
| 9. | MOV BL, SI | INVALID | BECAUSE THE THE SIZE OF THESE OPERANDS DIFFERE. Both operands must be same sized. |
| 10. | MOV AX, [BP+BX] | InValid | Because the assembler only allows base + index, base only ,index only ,index+base, any of them with displacement so base + base combination is not allowed. |
| 11. | MOV DX, [SI+DI] | Invalid | Because the assembler only allows base + index, base only ,index only ,index+base, any of them with displacement so index + index combination is not allowed. |
| 12. | ADD BX, [NUMBER + CX] | Invalid | CX is a general purpose register, and 8086 only allows bx, bp, si, and DI as memory address registers. |
| 13. | MOV [0X1234], BL | valid | Because If we write [0x1234] without specifying the size, the assembler infers the size from the register we are moving. |
| 14. | MOV AX, [BX+SI+4] | Valid | Because assembler allows this combination as [] contains base + index + displacement so it allowed |