Assignment 2      		University of Central Punjab

NAME: Muhammad Imran Butt

Reg no: L1F24BSCS0546

Total Marks: 20                                              Submission Date: 07/11/2025
Topics Covered: Data Types, Variable Declaration, ALU Operations, Arrays & Loops, Addressing Modes

Q1. Coding Task – ALU Operations and Variables (10Marks)

Write an 8086 assembly language program to:

- Declare two byte variables, NUM1 and NUM2, initialized with values 25H and 15H.

- Perform the following operations using ALU instructions:

  - Add the two numbers and store result in SUM.

  - Subtract NUM2 from NUM1 and store result in DIFF.

- Display both results using the register indirect addressing method.

(Hint: Use MOV, ADD, SUB instructions and memory variables.)

ANSWER:

CODE:

.model small

.stack 100h

.data

NUM1 DB 25H

NUM2 DB 15H

SUM DB 0

DIFF DB 0

.CODE

MOV AX,@DATA

MOV DS,AX

MOV DL,[NUM1]

ADD DL,[NUM2]

MOV [SUM],DL

MOV DL,[NUM1]

SUB DL,[NUM2]

MOV [DIFF],DL

MOV BX, OFFSET SUM

MOV AH,0H

MOV AL,[BX] ;SUM

MOV BX,OFFSET  DIFF

MOV DL,[BX];DIFFERENCE

.EXIT

SCREENSHOT:

NUM1 NUM2 SUM DIFF

### Q2. Coding Task – Arrays and Loops (10Marks)

Write an assembly program that:

- Declares an array of 5 elements (word type).

- Uses a label and LOOP instruction to:

  - Multiply each element by 2 and store it in the same array.

- Access array elements using Base + Index addressing mode.

- Finally, print all elements to verify results.

(Hint: Use SI, BX, and LOOP instruction effectively.)

CODE:

.MODEL SMALL

.DATA

ARR DW 1001,2002,3003,4004,5004

.CODE

MOV AX,@DATA

MOV DS, AX

MOV CX,5

MOV SI,0

MOV DI,0

MOV BX, OFFSET ARR

MY_LOOP:

MOV AX,2; TO MULTIPY (MULTIPIER)

MOV DI,[BX + SI]

MUL DI

MOV [BX + SI],AX

ADD SI,2

LOOP MY_LOOP

.EXIT